#include "BluetoothSerial.h"

#include <ESP32Servo.h>

BluetoothSerial SerialBT;

int motor1F = 18;  // MOTOR1 FORWARD

int motor1B = 19;  // MOTOR1 BACKWARD

int motor2F = 15;

int motor2B = 16;

#define PIN_Servo1 13  // SG90 DATA PIN

#define PIN_Servo2 14  // Valid GPIO pin

#define trig_F_Right 22

#define echo_F_Right 23

const int irSensorPin = 2; // OUT pin of the IR sensor

#define SOUND_SPEED 0.034

long duration_us;

float distanceCm;

bool autoparking = false;

int autoparkingState = 0; // Autoparking state

int sensorValue = 0;      // Stores IR sensor readings

Servo myServo1;

Servo myServo2; // Retained in case it's needed

unsigned long previousMillis = 0; // For non-blocking timing

unsigned long backMillis = 0;     // For non-blocking back logic

const unsigned long backInterval = 50; // Time interval for IR sensor checks

char lastCommand = '\0';  // Last Bluetooth command

bool isBacking = false;   // Indicates if the car is currently backing up

void setup() {

Serial.begin(115200);

SerialBT.begin("autoparking_car_3 test");  // Device name

SerialBT.println("Connected...");

myServo1.attach(PIN_Servo1);

myServo1.write(95); // Neutral position

myServo2.attach(PIN_Servo2);

pinMode(motor1F, OUTPUT);

pinMode(motor1B, OUTPUT);

pinMode(motor2F, OUTPUT);

pinMode(motor2B, OUTPUT);

pinMode(trig_F_Right, OUTPUT);

pinMode(echo_F_Right, INPUT);

pinMode(irSensorPin, INPUT); // IR sensor pin

}

void readBluetoothData() {

if (SerialBT.available()) {

char c = SerialBT.read();

SerialBT.println(c);

if (c == lastCommand) return; // Ignore repeated commands

if (c == '0' && autoparking) return; // Ignore '0' during autoparking

lastCommand = c;

switch (c) {

case '1': Left(); break;

case '2': Leftup(); break;

case '3': Ahead(); break;

case '4': Rightup(); break;

case '5': Right(); break;

case '6': Slow(); break;

case '7': Normal(); break;

case '8': Fast(); break;

case 'A': Forward(); break;

case 'a': Forwards(); break;

case 'B': StartBack(); break;

case 'b': Stop(); isBacking = false; break;

case 'C': Stop(); break;

case '0': Autoparking(); break;

}

}

}

void Ahead() { myServo1.write(95); }

void Left() { myServo1.write(40); }

void Right() { myServo1.write(140); }

void Leftup() { myServo1.write(65); }

void Rightup() { myServo1.write(125); }

void Slow() { myServo2.write(0); }

void Normal() { myServo2.write(90); }

void Fast() { myServo2.write(180); }

void Forward() {

digitalWrite(motor1F, HIGH); digitalWrite(motor1B, LOW);

digitalWrite(motor2F, HIGH); digitalWrite(motor2B, LOW);

}

void Forwards() {

digitalWrite(motor1F, LOW); digitalWrite(motor1B, LOW);

digitalWrite(motor2F, LOW); digitalWrite(motor2B, LOW);

}

void StartBack() {

Serial.println("Starting Backward Movement");

isBacking = true;

}

void Back() {

// Continually check the IR sensor while backing up

unsigned long currentMillis = millis();

if (currentMillis - backMillis >= backInterval) {

backMillis = currentMillis;

sensorValue = digitalRead(irSensorPin); // Read IR sensor

Serial.print("IR Sensor Value: ");

Serial.println(sensorValue);

if (sensorValue == LOW) { // Object detected

Serial.println("Object detected! Stopping...");

digitalWrite(motor1F, HIGH); digitalWrite(motor1B, LOW);

digitalWrite(motor2F, HIGH); digitalWrite(motor2B, LOW);

delay(150);

Stop(); // Call Stop to stop the car

isBacking = false; // Stop backing logic

} else {

// Continue moving backward

digitalWrite(motor1F, LOW); digitalWrite(motor1B, HIGH);

digitalWrite(motor2F, LOW); digitalWrite(motor2B, HIGH);

}

}

}

void Stop() {

digitalWrite(motor1F, LOW); digitalWrite(motor1B, LOW);

digitalWrite(motor2F, LOW); digitalWrite(motor2B, LOW);

}

void Autoparking() {

autoparking = true;

autoparkingState = 1;

Serial.println("Starting autoparking...");

}

void processAutoparking() {

if (!autoparking) return;

unsigned long currentMillis = millis();

switch (autoparkingState) {

case 1: // Measure distance

digitalWrite(motor1F, HIGH); digitalWrite(motor1B, LOW);

digitalWrite(motor2F, HIGH); digitalWrite(motor2B, LOW);

digitalWrite(trig_F_Right, LOW);

delayMicroseconds(2);

digitalWrite(trig_F_Right, HIGH);

delayMicroseconds(10);

digitalWrite(trig_F_Right, LOW);

duration_us = pulseIn(echo_F_Right, HIGH);

distanceCm = duration_us * SOUND_SPEED / 2;

if (distanceCm <= 3.5) autoparkingState = 2;

break;

case 2: // First phase of movement

if (currentMillis - previousMillis >= 500) {

//previousMillis = currentMillis;

Forward();

delay(400);

myServo1.write(140);

autoparkingState = 3;

}

break;

case 3: // Second phase of movement

if (currentMillis - previousMillis >= 1200) {

//previousMillis = currentMillis;

Back();

delay(1200);

myServo1.write(40);

autoparkingState = 4;

}

break;

case 4: // Final phase

sensorValue = digitalRead(irSensorPin);

if (sensorValue == LOW) {

Stop();

Serial.println("Autoparking complete!");

myServo1.write(95);

autoparking = false;

autoparkingState = 0;

} else {

Back();

}

break;

}

}

void loop() {

readBluetoothData();

// Process backward movement if triggered

if (isBacking) {

Back();

}

// Process autoparking if active

processAutoparking();

}