//ARDUINO SET UP

#include "BluetoothSerial.h"

#include <ESP32Servo.h>

#include <Wire.h>

//BLUETOOTH SET UP

#if !defined(CONFIG_BT_ENABLED) || !defined(CONFIG_BLUEDROID_ENABLED)

#error Bluetooth is not enabled! Please run `make menuconfig` to and enable it//防止錯誤的檢查指令

#endif

BluetoothSerial SerialBT;

//MOTOR DRIVE L293D SET UP

const int motorF = 5;  //MOTOR FORWARD

const int motorB = 18;  //MOTOR BACKWARD

//int enable_motor = 14;  //MOTER DRIVE SIGNL

//SERVO MOTOR SG90 SET UP

#define PIN_Servo 13  //SG90 DATA PIN

Servo myServo;

//ULTRASOUND HC-SR05 SET UP

#define trig_F_Right 22  //TRIGGER PIN OF THE RIGHT FRONT AIMING SENSOR

#define echo_F_Right 23  //ECHO PIN OF THE RIGHT FRONT ULTRASONIC SENSOR

#define trig_Back 27      //TRIGGER PIN OF THE BACK AIMING SENSOR

#define echo_Back 26     //ECHO PIN OF THE BACK ULTRASONIC SENSOR

//VARIABLES REQUIRED FOR ULTRASONIC RANGING

#define SOUND_SPEED 0.034

#define CM_TO_INCH 0.393701

long R_duration_us, B_duration_us;

float R_distanceCm, B_distanceCm;

float R_distanceInch, B_distanceInch;

bool autoparking = false;

bool backspace = false;

//ESP32-WROOM-32 SET UP

void setup() {

Serial.begin(115200);               //BAO RATE

SerialBT.begin("autoparking_car_3");  //DEVICE NAME

SerialBT.println("Connected...");   //CHECK BLUETOOTH CONNECT

pinMode(motorF, OUTPUT);

pinMode(motorB, OUTPUT);

//  pinMode(enable_motor, OUTPUT);

myServo.attach(PIN_Servo);

pinMode(trig_F_Right, OUTPUT);

pinMode(echo_F_Right, INPUT);

pinMode(trig_Back, OUTPUT);

pinMode(echo_Back, INPUT);

}

//BLUETOOTHDATA SET UP

void readBluetoothData() {

if (SerialBT.available()) {

char c = SerialBT.read();

SerialBT.println(c);

switch (c) {

case '8':

Forward();

break;

case '2':

Back();

break;

case '4':

Left();

break;

case '6':

Right();

break;

case '0':

Autoparking();

break;

case '9':

Stop();

break;

}

}

}

//CAR FORWARD PROGRAM

void Forward(void) {

Serial.println("Forward");

myServo.write(90);

delay(100);

//analogWrite(enable_motor,255);

digitalWrite(motorF, HIGH);

digitalWrite(motorB, LOW);

}

//CAR BACK PROGRAM

void Back(void) {

Serial.println("Backward");

//analogWrite(enable_motor,255);

digitalWrite(motorF, LOW);

digitalWrite(motorB, HIGH);

}

//CAR STOP PROGRAM

void Stop(void) {

Serial.println("Stop");

myServo.write(90);

//analogWrite(enable_motor,0);

digitalWrite(motorF, LOW);

digitalWrite(motorB, LOW);

}

//CAR LEFT PROGRAM

void Left(void) {

Serial.println("Left");

myServo.write(40);

delay(10);

}

//CAR RIGHT PROGRAM

void Right(void) {

Serial.println("Right");

myServo.write(140);

delay(10);

}

//CAR SELFPARKING PROGRAM

void Autoparking(void) {

autoparking = true;

while (autoparking == true) {

Forward();

digitalWrite(trig_F_Right, LOW);

delayMicroseconds(2);

digitalWrite(trig_F_Right, HIGH);

delayMicroseconds(10);

digitalWrite(trig_F_Right, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

R_duration_us = pulseIn(echo_F_Right, HIGH);

// Calculate the distance

R_distanceCm = R_duration_us * SOUND_SPEED / 2;

// Convert to inches

R_distanceInch = R_distanceCm * CM_TO_INCH;

Serial.print("右前 (cm): ");

Serial.println(R_distanceCm);

delayMicroseconds(10);

if (R_distanceCm <= 3.5 ) {

Stop();

delay(10);

Forward();

delay(500);

Stop();

Forward();

delay(300);

Stop();

delay(100);

myServo.write(140);

delay(100);

Back();

delay(2600);

Stop();

delay(100);

myServo.write(40);

delay(1900);

backspace = true;

while (backspace == true) {

R_distanceCm = 3.5 ;

digitalWrite(trig_F_Right, LOW);

Back();

digitalWrite(trig_Back, LOW);

delayMicroseconds(2);

digitalWrite(trig_Back, HIGH);

delayMicroseconds(10);

digitalWrite(trig_Back, LOW);

// Reads the echoPin, returns the sound wave travel time in microseconds

B_duration_us = pulseIn(echo_Back, HIGH);

// Calculate the distance

B_distanceCm = B_duration_us * SOUND_SPEED / 2;

// Convert to inches

B_distanceInch = B_distanceCm * CM_TO_INCH;

Serial.print("後方 (cm): ");

Serial.println(B_distanceCm);

delayMicroseconds(10);

if (B_distanceCm <= 1) {

Stop();

delay(1000);

backspace = false;

autoparking = false;

break;

}

}

}

}

}

//ESP32-WROOM-32 SET UP

void loop() {

readBluetoothData();

delay(10);

}